n

Name: _____________________________________________

Company: __________________________________________

- For one of the motors on your initial list of important motors, have an electrician measure the volts (phase to phase) and the amps on each phase.

- Calculate the average voltage and the voltage deviation according to the formula in the slide presentation.

- Use the “percent load estimation calculator” in MEASUR software to calculate the % load and the electrical input power and compare the voltage unbalance from MEASUR to your manual calculation.

- Skim or read the DOE documents on best practices for motor repair and summarize the motor repair vs replace policy for your facility.  Consult with managers and colleagues as necessary.

- Open the “Replace vs Rewind” calculator in MEASUR software and use the “generate example” button to see how it works. 
“Extra credit” - dig up some data on a motor that was recently rewound and use your data in the “Rewind vs Replace” calculator.

- What was the cost of the rewind?

- What would have been the cost of replacing it with a new motor? (may need to call your supplier)?

- Assuming either ½ or 1% additional losses, calculate the simple payback of buying a new motor vs rewinding the old one.